Lab wireless - Guideline

Topics

A. Wireless topologies

B.  802.11 protocol

A. Wireless topologies

WWAN:

- GSM, GPRS, CDMA, TDMA, EDGE, MBWA,

WMAN:

- 802.16 WiMAX

WPAN:

- Bluetooth, ZigBee

B.  802.11 protocol

- 802.11 physical layer

- RF (FHSS, DSSS), IR

- RF:

- 5GHz Unlicensed National Information Infrastructure (U-NII),

- 2.4GHz Industrial, Scientific, and Medical (ISM)

- 802.11 a,b,g,n,ac

Ad-hoc Mode

- Independent Basic Service Set (IBSS)

Infrastructure Mode

- Access Point (AP) – layer 2 device; role: bridge traffic

- AP and clients: BSS (Basic Service Set)

- Several BSS connected: ESS (Extended Service Set)

- SSID (Service Set Identifier:)

Example of wired/wireless topology using Wireless Routers and APs:

POE (Power over Ethernet)

www.tp-link.us

Wifi Extender

www.edimax.com

Power line adapters (Powerline AV2/ HomePlug AV2 standard)

http://tekeye.biz/

www.tp-link.pt

Using VLANs on a network – different VLAN for WiFi

Laboratory test configuration:

| Device |  | IP Address | Subnet mask | Default Gateway |
| --- | --- | --- | --- | --- |
| Router | Internet | DHCP | DHCP | DHCP |
|  | Wireless | 192.168.0.1 | 255.255.255.0 | - |
| PCs, Laptop |  | 192.168.0.x Dynamic (DHCP) | 255.255.255.0 Dynamic (DHCP) | 192.168.0.1 Dynamic (DHCP) |

| Device | SSID | Standard Channel | Wireless Sec. Protocol | Wireless Password | Remote Management Password |
| --- | --- | --- | --- | --- | --- |
| Router Wireless | Wi-Fi | 11 | WPA2 Personal | Cisco1234 | Pass1234 |

Copy the following configuration to router ISP, in the configuration mode:

hostname ISP

!

interface GigabitEthernet0/0

ip address 1.1.1.1 255.0.0.0

no shutdown

!

ip dhcp excluded-address 1.1.1.1

!

ip dhcp pool WiFi

network 1.0.0.0 255.0.0.0

default-router 1.1.1.1

dns-server 8.8.8.8

Test the connectivity.

- ping <target IP> and tracert <target IP>

Configure a wireless router:

Connect to a wireless AP from Laptop0: